Gas Safety Policy

| ODBST Level 1 Statutory Policy: | ALL Schools require this policy with no changes allowed to core text. No changes are necessary to personalise this with school name and branding, as this is a Trust level policy for use, without change, by all schools, except where a school contact is required as identified in the content of the policy. LGBs will note adoption in LGB meetings. Review will take place at Trust level, and schools will be notified of updates and review dates as necessary. |
| --- | --- |
| Other related ODBST policies and procedures: | ODBST Health and Safety Policy |
| Committee responsible: | FRAPP |
| Approved by: | FRAPP |
| Date Approved: | 27/11/2025 |
| Date for Next Review: | 26/11/2028 |

In reviewing this policy the Trust Board has had regards to the Equality act 2010 and carried out an equality impact assement. It is satisfied that no group with a protected charecteristic will be unfairly disadvantaged

# Gas Safety Policy

## Applicable Legislation

Gas Safety (Installation and Use) Regulations 1998

## Purpose

Most Oxford Diocesan Bucks Schools Trust buildings have access to a gas supply which is either fed from the mains or in some cases LPG (Propane & Butane). The effects of poor or faulty installation, misuse or neglect of gas systems can be extremely serious. It is, therefore, particularly important for managers to consider potential risks when dealing with the gas installation and gas fueled equipment.

The risks associated with the use of gas are:

- Fire

- Explosion

- Poisoning by the by-products of combustion, such as carbon monoxide

## Scope

This procedure applies to all corporate buildings and schools.

## Definitions

A ‘gas appliance’ is defined as an appliance designed for use by a consumer of gas for heating, lighting, cooking or other purpose for which gas can be used.

## General Requirements

The main requirements of regulations are:

- To design gas installations safely and with a view to anticipated use.

- To maintain gas installations in a safe condition.

- To have an annual safety-check in accordance with the Gas Safety (Installation and Use) Regulations 1998on fixed gas appliances, pipework and ventilation systems. A Commercial Safety Certificate CP42 must be obtained and stored securely. The Certificate must detail the date of inspection, the engineer’s credentials, appliances tested, any defects identified, and remedial actions taken.

- To allow only engineers (those who are on the Gas Safe Register) to work on gas installations and appliances of the categories for which they are competent.

Other requirements which gas engineers must comply with are:

- To supply gas equipment which complies with current manufacturers’ standards.

- To use safe systems of work when installing and maintaining gas equipment.

## Competent Engineer

All work on gas pipe work and appliances must only be carried out by Gas Safe Registered engineers and hold a valid certificate of competence relevant to the work involved.

Prior to an engineer undertaking any work involving gas appliances a check to ensure that he/she is fully qualified to undertake the task must be made. Insist on inspecting registration cards; the reverse of the card will also confirm what specific jobs the person is competent to undertake.

Front	Back

## If the engineer is unable or unwilling to show proof of registration do not permit any work to proceed and contact Gas Safe to report the incident.

Premises Managers’ Responsibilities

Schools

All schools in ODBST will need to make their own arrangements for gas related installation, testing and servicing with a competent Gas Safe registered contractor. This will include curriculum equipment, for example gas cookers in teaching areas, heat treatment equipment in Design and Technology, etc.

N.B. Bunsen Burners in educational establishment/settings are excluded under these regulations.

Records should be kept of the date of inspection, defects identified, and remedial action taken. A commissioning   certificate should be obtained before re-use. Copies should be retained and made available to the Trust on   request. Records should be stored in Smartlog, filed under Gas safety and dated.

No person should carry out any work on a gas installation or equipment unless they are competent, have been authorised to do so and are on the Gas Safe Register. (wwww.hse.gov.uk/gas)

Any incident involving a gas leak, fire, carbon monoxide poisoning or explosion should be reported immediately to the  National Gas Emergency number 9UK) 0800 111 999    (Free to call and 24 hours 7 days a week) and OBDST central team immediately.

Handsam (Trust Competent Advisor) : 0333 207 0737

Chief Operating Officer 07391 416990 or  Operations Partner: 07827 827791

## Employees’ Responsibilities

All gas appliances must be examined daily before use. The following checklist will help, so far as is reasonably practicable, the safe use of gas equipment:

## Before use:

  - Ensure all ventilation grilles/air bricks are free from obstruction.

  - Turn on any extraction/ventilation fans.

  - Visually inspect each gas appliance to make sure the gas control valves are in the ‘off’ position.

## After Use:

  - Turn off the gas control valve at each individual appliance.

  - Turn off the main gas supply to the kitchen, workshop or laboratory.

  - Turn the temperature control of fryers and ovens to 0°C (zero).

  - Visually inspect each gas appliance to make sure that gas control valves are in the ‘off’ position.

  - Turn off all extraction/ventilation fans.

If you detect any fault report it to your manager immediately!

## Emergency Procedure

If you smell gas:

    - Turn off main gas control valve.

    - Open all doors and windows to ventilate the area.

    - Eliminate all sources of ignition.

    - Do not operate electrical equipment or switches either on or off.

    - Extinguish any naked flames.

    - Evacuate the building if necessary

If you smell gas contact your energy supplier immediately using their emergency number.

National Gas Emergency number 9UK) 0800 111 999    (Free to call and 24 hours 7 days a week)

This number can also be used for reporting suspected carbon monoxide incidents and damaged gas pipes (even if no leak is visible.

For further information and advice:

| ADD NAME OF GAS SUPPLIER AND PHONE NUMBER FOR SCHOOL | xxxx | Tel: xxx |
| --- | --- | --- |
| OCC Health and Safety Team | info@handsam.co.uk | Tel: 0333 207 0737 |
| Health and Safety Executive (HSE) | http://www.hse.gov.uk/gas/ Advice on requirements for notifying and reporting gas incidents under the Reporting of Injuries, Diseases and Dangerous Occurrences Regulations 2013 (RIDDOR) and the Gas Safety (Management) Regulations 1996 (GSMR) |  |
| Gas Safe Register | http://www.gassaferegister.co.uk/ |  |